Ramsey County Project to Convert 1991 Aerial Flight Negatives to Geo-referenced, Orthomosaiced Digital Imagery

Project Scope

Ramsey County has approximately 538, 9 inch x 9 inch, 1991 aerial photograph negatives to be converted into geo-referenced , orthomosaiced digital orthophoto images.  The finished products will be edgematched, high resolution, high accuracy, georeferenced, orthomosaiced, GeoTiff and JPEG2000 digital images.

Data and Equipment Supplied by Ramsey County

- 2003 Aerial Photography (Geo Tiff Image Format)

- 1991 Aerial Photograph Negatives (Approximately 538)

- 1991 Flight index  (Paper copy and Adobe PDF)

- Ramsey County Coordinate System NAD 83, 1986 Adjustment, Non-HARN

- Ramsey County Monuments ASCII text file and DBF

- 2003 Ortho Tile Index Grid (Use for this project)?

- 1998?  DNR 5 meter ASCII XYZ DEM for Ramsey County

- 1996 Bridge R2004 DWG

- 1996 Road edges R2004 DWG

- 1996 Structures R2004 DWG

- 1991 Aerial Flight Camera Calibration

- USB External Hard drive

Data, Equipment and Labor Supplied by Dudley Thomson Mapping Corporation Inc.

- All equipment, computer hardware and software

- Personnel (staff) to complete project

Deliverables

- 1991 Aerial Flight Negatives (returned to Ramsey County after scanning)

- Final Orthophoto images in GeoTiff and JPEG2000 image formats

- Final products shall conform to the Ramsey County Coordinate System.

- Supplied USB External hard drive (with Final Ortho images loaded)

- Metadata with statement of accuracy.

Project Accuracy

This project shall comply with the National Standards for Spatial Data Accuracy (NSSDA) whereby 95% of the images shall not exceed a horizontal positional error of 2.50 feet and sufficient for 1:1200 scale mapping.

Image Products

Images shall to highest degree possible eliminate image distortion, misalignment, image smearing, image blurring and tonal imbalances.  This includes bridges, bluffs and other elevated feature distortions.

Project Payment

Total project cost and payment to Dudley Thompson Mapping Corporation Inc. shall not exceed $15,000.00

Payments will be made as follows:

50% Project Commencement

40% Completion of scanning

10% Upon the return and acceptance of the finished products and flight negatives

Project Timeline

The project shall be completed in approximately 120 calendar days.

Project Management

Contractor will work closely with The Ramsey County Surveyor’s Office.  The following individuals will serve as the project contacts:

Michael Fiebiger, Ramsey County Surveyor

Email:  micheal.fibiger@co.ramsey.mn.us   Phone:  651-266-7171

Curtis Peterson, GIS Supervisor

Email: curtis.peterson@co.ramsey.mn.us  Phone: 651-266-7172